*SWF1

vlan 10

name GiaoDich

vlan 11

name Hanhchinh

interface g0/1

switchport mode access

switchport access vlan 10

interface f0/3

switchport mode access

switchport access vlan 10

interface f0/6

switchport mode access

switchport access vlan 10

interface f0/4

switchport mode access

switchport access vlan 11

interface f0/5

switchport mode access

switchport access vlan 11

interface range f0/1-2

switchport mode trunk

switchport trunk allowed vlan 10,11,20,21,30,31,32,33,40

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

ip dhcp snooping vlan 10-11

ip dhcp snooping

spanning-tree mode pvst

spanning-tree portfast bpduguard default

spanning-tree extend system-id

interface FastEthernet0/1

switchport trunk allowed vlan 10-11,20-21,30-33,40

ip dhcp snooping trust

switchport mode trunk

interface FastEthernet0/2

switchport trunk allowed vlan 10-11,20-21,30-33,40

ip dhcp snooping trust

switchport mode trunk

interface FastEthernet0/3

switchport access vlan 10

switchport mode access

switchport port-security

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0001.6469.7E4E

interface FastEthernet0/4

switchport access vlan 11

switchport mode access

switchport port-security

switchport port-security mac-address sticky

interface FastEthernet0/5

switchport access vlan 11

switchport mode access

switchport port-security

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0006.2A3D.1D54

interface FastEthernet0/6

switchport access vlan 10

switchport mode access

switchport port-security

switchport port-security mac-address sticky

interface GigabitEthernet0/1

switchport access vlan 10

switchport trunk native vlan 33

switchport trunk allowed vlan 10-11,20-21,30-33,40

switchport mode trunk

logging trap debugging

logging 192.168.3.4

ntp server 192.168.3.3

*SWF2

vlan 20

name DichVu

vlan 21

name TaiChinh

interface g0/1

switchport mode access

switchport access vlan 20

interface f0/3

switchport mode access

switchport access vlan 20

interface f0/4

switchport mode access

switchport access vlan 20

interface f0/5

switchport mode access

switchport access vlan 21

interface f0/6

switchport mode access

switchport access vlan 21

interface range f0/1-2

switchport mode trunk

switchport trunk allowed vlan 10,11,20,21,30,31,32,33,40

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

ip dhcp snooping vlan 20-21

ip dhcp snooping

spanning-tree mode pvst

spanning-tree portfast bpduguard default

spanning-tree extend system-id

interface FastEthernet0/1

switchport trunk allowed vlan 10-11,20-21,30-33,40

ip dhcp snooping trust

switchport mode trunk

interface FastEthernet0/2

switchport trunk allowed vlan 10-11,20-21,30-33,40

ip dhcp snooping trust

switchport mode trunk

interface FastEthernet0/3

switchport access vlan 20

switchport mode access

switchport port-security

switchport port-security mac-address sticky

switchport port-security mac-address sticky 00D0.D372.C7AD

interface FastEthernet0/4

switchport access vlan 20

switchport mode access

switchport port-security

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0090.2B91.1772

interface FastEthernet0/5

switchport access vlan 21

switchport mode access

switchport port-security

switchport port-security mac-address sticky

interface FastEthernet0/6

switchport access vlan 21

switchport mode access

switchport port-security

switchport port-security mac-address sticky

interface GigabitEthernet0/1

switchport access vlan 20

switchport trunk native vlan 33

switchport trunk allowed vlan 10-11,20-21,30-33,40

switchport mode trunk

logging trap debugging

logging 192.168.3.4

ntp server 192.168.3.3

*SWF3

vlan 30

name KinhDoanh

vlan 31

name GiamDoc

interface f0/5

switchport mode access

switchport access vlan 30

interface f0/6

switchport mode access

switchport access vlan 30

interface g0/1

switchport mode access

switchport access vlan 31

interface f0/3

switchport mode access

switchport access vlan 31

interface f0/4

switchport mode access

switchport access vlan 31

interface range f0/1-2

switchport mode trunk

switchport trunk allowed vlan 10,11,20,21,30,31,32,33,40

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

ip dhcp snooping vlan 30-31

ip dhcp snooping

spanning-tree mode pvst

spanning-tree portfast bpduguard default

spanning-tree extend system-id

interface FastEthernet0/1

switchport trunk allowed vlan 10-11,20-21,30-33,40

ip dhcp snooping trust

switchport mode trunk

interface FastEthernet0/2

switchport trunk allowed vlan 10-11,20-21,30-33,40

ip dhcp snooping trust

switchport mode trunk

interface FastEthernet0/3

switchport access vlan 31

switchport mode access

switchport port-security

switchport port-security mac-address sticky

switchport port-security mac-address sticky 000C.8582.5560

interface FastEthernet0/4

switchport access vlan 31

switchport mode access

switchport port-security

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0001.646B.D4ED

interface FastEthernet0/5

switchport access vlan 30

switchport mode access

switchport port-security

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0005.5E9D.522E

interface FastEthernet0/6

switchport access vlan 30

switchport mode access

switchport port-security

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0090.2B93.7D45

interface GigabitEthernet0/1

switchport access vlan 31

switchport trunk native vlan 33

switchport trunk allowed vlan 10-11,20-21,30-33,40

switchport mode trunk

logging trap debugging

logging 192.168.3.4

ntp server 192.168.3.3

*SW_F4

vlan 32

name KyThuat

vlan 33

name QuanLy

interface g0/1

switchport mode access

switchport access vlan 32

interface f0/3

switchport mode access

switchport access vlan 32

interface f0/4

switchport mode access

switchport access vlan 33

interface f0/5

switchport mode access

switchport access vlan 33

interface f0/6

switchport mode access

switchport access vlan 33

interface g0/2

switchport mode access

switchport access vlan 33

interface range f0/1-2

switchport mode trunk

switchport trunk allowed vlan 10,11,20,21,30,31,32,33,40

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

ip dhcp snooping vlan 32

ip dhcp snooping

spanning-tree mode pvst

spanning-tree portfast bpduguard default

spanning-tree extend system-id

interface FastEthernet0/1

switchport trunk allowed vlan 10-11,20-21,30-33,40

ip dhcp snooping trust

switchport mode trunk

interface FastEthernet0/2

switchport trunk allowed vlan 10-11,20-21,30-33,40

ip dhcp snooping trust

switchport mode trunk

interface FastEthernet0/3

switchport access vlan 32

switchport trunk native vlan 33

switchport mode access

switchport port-security

switchport port-security mac-address sticky

switchport port-security mac-address sticky 000D.BD6E.0895

interface FastEthernet0/4

switchport access vlan 33

switchport mode access

switchport port-security

switchport port-security mac-address sticky

switchport port-security mac-address sticky 00D0.5862.1C99

interface FastEthernet0/5

switchport access vlan 33

switchport mode access

switchport port-security

switchport port-security mac-address sticky

switchport port-security mac-address sticky 0001.C741.133C

interface FastEthernet0/6

switchport access vlan 33

switchport mode access

switchport port-security

switchport port-security mac-address sticky

interface FastEthernet0/7

switchport access vlan 33

switchport mode access

switchport port-security

switchport port-security mac-address sticky

interface GigabitEthernet0/1

switchport access vlan 32

switchport trunk native vlan 33

switchport trunk allowed vlan 10-11,20-21,30-33,40

switchport mode trunk

!

interface GigabitEthernet0/2

switchport access vlan 33

switchport trunk native vlan 33

switchport trunk allowed vlan 10-11,20-21,30-33,40

switchport mode trunk

logging trap debugging

logging 192.168.3.4

ntp server 192.168.3.3

*SW_DMZ

vlan 40

name DMZ

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

spanning-tree mode pvst

spanning-tree extend system-id

interface FastEthernet0/1

switchport access vlan 40

switchport trunk allowed vlan 10-11,20-21,30-33,40

switchport mode access

interface FastEthernet0/2

switchport access vlan 40

switchport mode access

interface FastEthernet0/3

switchport access vlan 40

switchport mode access

interface FastEthernet0/4

switchport access vlan 40

switchport mode access

interface FastEthernet0/5

switchport access vlan 40

switchport mode access

ip default-gateway 192.168.2.1

ntp server 192.168.3.3

*SWL3_1

! bật IP routing nếu là L3 switch

ip routing

vlan 10

name GiaoDich

vlan 11

name Hanhchinh

vlan 20

name DichVu

vlan 21

name TaiChinh

vlan 30

name KinhDoanh

vlan 31

name GiamDoc

vlan 32

name KyThuat

vlan 33

name QuanLy

vlan 40

name DMZ

int range f0/3-6

switch port trunk encapsulation dot1q

switch port mode trunk

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

ip routing

crypto isakmp client configuration group GroupVPN

spanning-tree mode pvst

interface FastEthernet0/1

no switchport

ip address 10.0.0.2 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 CISCO123

interface FastEthernet0/2

no switchport

ip address 10.0.0.14 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 CISCO123

interface FastEthernet0/3

switchport trunk encapsulation dot1q

switchport mode trunk

interface FastEthernet0/4

switchport trunk encapsulation dot1q

switchport mode trunk

interface FastEthernet0/5

switchport trunk encapsulation dot1q

switchport mode trunk

interface FastEthernet0/6

switchport trunk encapsulation dot1q

switchport mode trunk

interface Vlan10

mac-address 0004.9ac5.c901

ip address 192.168.0.2 255.255.255.224

ip helper-address 192.168.1.154

ip helper-address 192.168.3.3

standby 10 ip 192.168.1.1

standby 10 priority 110

standby 10 preempt

interface Vlan11

mac-address 0004.9ac5.c902

ip address 192.168.1.34 255.255.255.224

ip helper-address 192.168.1.154

ip helper-address 192.168.3.3

standby 11 ip 192.168.1.33

standby 11 priority 110

standby 11 preempt

interface Vlan20

mac-address 0004.9ac5.c903

ip address 192.168.1.66 255.255.255.224

ip helper-address 192.168.1.154

ip helper-address 192.168.3.3

standby 20 ip 192.168.1.65

standby 20 preempt

interface Vlan21

mac-address 0004.9ac5.c904

ip address 192.168.1.98 255.255.255.224

ip helper-address 192.168.1.154

ip helper-address 192.168.3.3

standby 21 ip 192.168.1.97

standby 21 preempt

interface Vlan30

mac-address 0004.9ac5.c905

ip address 192.168.1.129 255.255.255.248

ip helper-address 192.168.1.154

ip helper-address 192.168.3.3

interface Vlan31

mac-address 0004.9ac5.c906

ip address 192.168.1.137 255.255.255.248

ip helper-address 192.168.1.154

ip helper-address 192.168.3.3

interface Vlan32

mac-address 0004.9ac5.c907

ip address 192.168.1.145 255.255.255.248

ip helper-address 192.168.1.154

ip helper-address 192.168.3.3

interface Vlan33

mac-address 0004.9ac5.c908

ip address 192.168.3.1 255.255.255.240

ip helper-address 192.168.1.154

interface Vlan40

mac-address 0004.9ac5.c909

no ip address

ip helper-address 192.168.1.154

router ospf 1

router-id 1.1.1.1

log-adjacency-changes

network 10.0.0.4 0.0.0.3 area 0

network 10.0.0.0 0.0.0.3 area 0

network 192.168.1.0 0.0.0.255 area 0

network 10.0.0.12 0.0.0.3 area 0

network 192.168.3.0 0.0.0.15 area 0

ip classless

ip route 0.0.0.0 0.0.0.0 10.0.0.1

ip route 0.0.0.0 0.0.0.0 10.0.0.5

ip route 192.168.2.0 255.255.255.248 10.0.0.17

ip route 0.0.0.0 0.0.0.0 10.0.0.13

ip route 192.168.2.0 255.255.255.248 10.0.0.21 10

snmp-server community read RO

snmp-server community write RW

logging trap debugging

logging 192.168.3.4

ntp server 192.168.3.3

*SWL3_ 2

service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

ip routing

spanning-tree mode pvst

interface FastEthernet0/1

no switchport

ip address 10.0.0.10 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 CISCO123

interface FastEthernet0/2

no switchport

ip address 10.0.0.6 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 CISCO123

interface FastEthernet0/3

switchport trunk encapsulation dot1q

switchport mode trunk

interface FastEthernet0/4

switchport trunk encapsulation dot1q

switchport mode trunk

interface FastEthernet0/5

switchport trunk encapsulation dot1q

switchport mode trunk

interface FastEthernet0/6

switchport trunk encapsulation dot1q

switchport mode trunk

interface Vlan10

mac-address 0060.5cb2.9501

ip address 192.168.1.3 255.255.255.224

ip helper-address 192.168.1.154

ip helper-address 192.168.3.3

standby 10 ip 192.168.1.1

standby 10 preempt

interface Vlan11

mac-address 0060.5cb2.9502

ip address 192.168.1.35 255.255.255.224

ip helper-address 192.168.1.154

ip helper-address 192.168.3.3

standby 11 ip 192.168.1.33

standby 11 preempt

interface Vlan20

mac-address 0060.5cb2.9503

ip address 192.168.1.67 255.255.255.224

ip helper-address 192.168.1.154

ip helper-address 192.168.3.3

standby 20 ip 192.168.1.65

standby 20 priority 110

standby 20 preempt

interface Vlan21

mac-address 0060.5cb2.9504

ip address 192.168.1.99 255.255.255.224

ip helper-address 192.168.1.154

ip helper-address 192.168.3.3

standby 21 ip 192.168.1.97

standby 21 priority 110

standby 21 preempt

interface Vlan30

mac-address 0060.5cb2.9505

ip address 192.168.1.129 255.255.255.248

ip helper-address 192.168.1.154

ip helper-address 192.168.3.3

interface Vlan31

mac-address 0060.5cb2.9506

ip address 192.168.1.137 255.255.255.248

ip helper-address 192.168.1.154

ip helper-address 192.168.3.3

interface Vlan32

mac-address 0060.5cb2.9507

ip address 192.168.1.145 255.255.255.248

ip helper-address 192.168.1.154

ip helper-address 192.168.3.3

interface Vlan33

mac-address 0060.5cb2.9508

ip address 192.168.3.1 255.255.255.240

ip helper-address 192.168.1.154

interface Vlan40

mac-address 0060.5cb2.9509

no ip address

ip helper-address 192.168.1.154

router ospf 1

router-id 1.1.1.2

log-adjacency-changes

network 10.0.0.8 0.0.0.3 area 0

network 10.0.0.12 0.0.0.3 area 0

network 192.168.1.0 0.0.0.255 area 0

network 10.0.0.4 0.0.0.3 area 0

network 192.168.3.0 0.0.0.15 area 0

ip classless

ip route 0.0.0.0 0.0.0.0 10.0.0.9

ip route 0.0.0.0 0.0.0.0 10.0.0.13

ip route 192.168.2.0 255.255.255.248 10.0.0.17

ip route 192.168.2.0 255.255.255.248 10.0.0.21 10

ip route 0.0.0.0 0.0.0.0 10.0.0.5

snmp-server community read RO

snmp-server community write RW

logging trap debugging

logging 192.168.3.4

ntp server 192.168.3.3

*Internet Router

interface GigabitEthernet0/0/0

ip address 8.8.8.1 255.255.255.0

interface Serial0/1/0

ip address 203.0.11.1 255.255.255.252

interface Serial0/1/1

ip address 203.0.12.1 255.255.255.252

router ospf 1

log-adjacency-changes

network 203.0.10.0 0.0.0.3 area 0

network 203.0.11.0 0.0.0.3 area 0

network 203.0.12.0 0.0.0.3 area 0

network 8.8.8.0 0.0.0.255 area 0

*Edge Router

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

aaa new-model

aaa authentication login UserVPN local

aaa authorization network GroupVPN local

username uservpn secret ciscovpn

crypto isakmp policy 100

encr aes 256

authentication pre-share

group 5

lifetime 3600

crypto isakmp client configuration group GroupVPN

key ciscogroupvpn

pool PoolVPN

crypto ipsec transform-set SetVPN esp-aes esp-sha-hmac

crypto dynamic-map DynamicVPN 100

set transform-set SetVPN

reverse-route

crypto map StaticMap client authentication list UserVPN

crypto map StaticMap isakmp authorization list GroupVPN

crypto map StaticMap client configuration address respond

crypto map StaticMap 20 ipsec-isakmp dynamic DynamicVPN

spanning-tree mode pvst

interface GigabitEthernet0/0/0

description Connect_to_ASA

ip address 203.0.10.1 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 CISCO123

interface Serial0/1/0

ip address 203.0.11.2 255.255.255.252

clock rate 2000000

crypto map StaticMap

router ospf 1

log-adjacency-changes

network 203.0.10.0 0.0.0.255 area 0

network 8.8.8.0 0.0.0.255 area 0

network 203.0.10.0 0.0.0.3 area 0

network 203.0.11.0 0.0.0.3 area 0

ip local pool PoolVPN 192.168.100.10 192.168.100.50

VPN_Router

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

spanning-tree mode pvst

interface GigabitEthernet0/0/0

ip address 192.168.10.1 255.255.255.0

interface Serial0/1/1

ip address 203.0.12.2 255.255.255.252

router ospf 1

log-adjacency-changes

network 203.0.12.0 0.0.0.3 area 0

network 192.168.10.0 0.0.0.255 area 0

ASA1_Internet

interface GigabitEthernet1/1

nameif inside1

security-level 100

ip address 10.0.0.1 255.255.255.252

ospf message-digest-key 1 md5 CISCO123

interface GigabitEthernet1/2

nameif inside2

security-level 100

ip address 10.0.0.5 255.255.255.252

ospf message-digest-key 1 md5 CISCO123

interface GigabitEthernet1/3

nameif outside

security-level 0

ip address 203.0.10.2 255.255.255.252

ospf message-digest-key 1 md5 CISCO123

object network INTERNAL

subnet 192.168.0.0 255.255.0.0

nat (inside2,outside) dynamic interface

object network L3_INTERFACES

subnet 10.0.0.0 255.255.255.252

nat (inside1,outside) dynamic interface

object network L3_INTERFACES_1

subnet 10.0.0.4 255.255.255.252

nat (inside2,outside) dynamic interface

object network LAN_INTERNET

subnet 192.168.0.0 255.255.0.0

nat (inside1,outside) dynamic interface

route outside 0.0.0.0 0.0.0.0 203.0.10.1 1

route outside 192.168.100.0 255.255.255.0 203.0.10.1 1

access-list NOSPOOF extended permit icmp any 192.168.0.0 255.255.0.0 echo-reply

access-list NOSPOOF extended permit icmp 192.168.100.0 255.255.255.0 any

access-list NOSPOOF extended permit ip 192.168.100.0 255.255.255.0 any

access-list NOSPOOF extended permit tcp any any

access-group NOSPOOF in interface outside

class-map inspection_default

match default-inspection-traffic

policy-map type inspect dns preset_dns_map

parameters

message-length maximum 512

policy-map global_policy

class inspection_default

inspect dns preset_dns_map

inspect ftp

inspect icmp

inspect tftp

service-policy global_policy global

router ospf 1

log-adjacency-changes

area 0 authentication message-digest

network 10.0.0.0 255.255.255.252 area 0

network 10.0.0.4 255.255.255.252 area 0

network 203.0.10.0 255.255.255.252 area 0

ASA2 – DMZ

interface GigabitEthernet1/1

nameif spine1

security-level 50

ip address 10.0.0.9 255.255.255.252

ospf message-digest-key 1 md5 CISCO

interface GigabitEthernet1/2

nameif spine2

security-level 50

ip address 10.0.0.13 255.255.255.252

ospf message-digest-key 1 md5 CISCO

interface GigabitEthernet1/3

nameif dmz

security-level 75

ip address 192.168.2.1 255.255.255.248

object network DNS

host 192.168.2.3

object network FTP

host 192.168.2.5

object network MAIL

host 192.168.2.4

object network WEB

host 192.168.2.2

route spine1 192.168.100.0 255.255.255.0 10.0.0.10 1

route spine2 192.168.100.0 255.255.255.0 10.0.0.14 1

access-list DMZ extended deny ip 192.168.1.0 255.255.255.224 object WEB

access-list DMZ extended deny ip 192.168.1.0 255.255.255.224 object MAIL

access-list DMZ extended deny ip 192.168.1.0 255.255.255.224 object FTP

access-list DMZ extended deny ip 192.168.1.0 255.255.255.224 object DNS

access-list DMZ extended permit tcp 192.168.1.32 255.255.255.224 any

access-list DMZ extended permit tcp 192.168.1.64 255.255.255.224 any

access-list DMZ extended permit tcp 192.168.1.96 255.255.255.224 any

access-list DMZ extended permit tcp 192.168.1.128 255.255.255.248 any

access-list DMZ extended permit tcp 192.168.1.136 255.255.255.248 any

access-list DMZ extended permit ip 192.168.1.144 255.255.255.248 object DNS

access-list DMZ extended permit ip 192.168.1.144 255.255.255.248 object WEB

access-list DMZ extended permit ip 192.168.1.144 255.255.255.248 object MAIL

access-list DMZ extended permit ip 192.168.1.144 255.255.255.248 object FTP

access-list DMZ extended permit ip 192.168.1.152 255.255.255.248 object DNS

access-list DMZ extended permit ip 192.168.1.152 255.255.255.248 object WEB

access-list DMZ extended permit ip 192.168.1.152 255.255.255.248 object MAIL

access-list DMZ extended permit ip 192.168.1.152 255.255.255.248 object FTP

access-list DMZ extended permit ip 192.168.3.0 255.255.255.240 object DNS

access-list DMZ extended permit ip 192.168.3.0 255.255.255.240 object WEB

access-list DMZ extended permit ip 192.168.3.0 255.255.255.240 object MAIL

access-list DMZ extended permit ip 192.168.3.0 255.255.255.240 object FTP

access-list DMZ extended permit tcp 192.168.100.0 255.255.255.0 any

access-list DMZ extended permit udp 192.168.1.32 255.255.255.224 object DNS eq domain

access-list DMZ extended permit udp 192.168.1.64 255.255.255.224 object DNS eq domain

access-list DMZ extended permit udp 192.168.1.96 255.255.255.224 object DNS eq domain

access-list DMZ extended permit udp 192.168.1.128 255.255.255.248 object DNS eq domain

access-list DMZ extended permit udp 192.168.1.136 255.255.255.248 object DNS eq domain

access-list DMZ extended permit udp 192.168.100.0 255.255.255.0 object DNS eq domain

access-group DMZ out interface dmz

class-map inspection_default

match default-inspection-traffic

policy-map type inspect dns preset_dns_map

parameters

message-length maximum 512

policy-map global_policy

class inspection_default

inspect dns preset_dns_map

inspect ftp

inspect tftp

service-policy global_policy global

router ospf 1

log-adjacency-changes

area 0 authentication message-digest

network 10.0.0.8 255.255.255.252 area 0

network 10.0.0.12 255.255.255.252 area 0

network 192.168.2.0 255.255.255.248 area 0